# Energy Model Update: Response to Feedback (October 2026)

Four experiments since the September brief, run on B200 and on our existing 110 H200/B200 serving runs. Each section covers what we ran and what it shows.

# 1. Do we saturate HBM? (proxy "blast HBM" kernels)

What we ran. Pure memory-read kernels on B200: 1–4 GB streams and an occupancy sweep from 3% to 92% of peak bandwidth. Each point is a 10-second steady loop with power read every 10 ms.

Figure 1. B200 power vs achieved bandwidth. Power rises in a straight line, 39 W + 125 pJ per byte, up to the 1000 W power limit. Peak-bandwidth kernels reach 92% of 8 TB/s, but only at the limit, with the SM clock throttled to about 1565 MHz.

What it shows.

HBM can be saturated, but the power limit gets there first. At full clock, 1000 W allows about 73% of peak bandwidth. Reaching 92% means slowing the cores to free up power.

Serving does not saturate it. Serving reaches 27–67% of peak, because most of each decode step is not spent reading memory. The fixed overhead is host/launch overhead plus a per-layer latency floor, and it does not shrink with a faster GPU. Bigger models amortize it better: Qwen2-72B on B200 reaches 62%. Qwen2-7B at batch 1:

|  | step time | reading the 14.1 GB of weights at peak bandwidth | fixed overhead | bandwidth used |
| --- | --- | --- | --- | --- |
| B200 | 5.4 ms | 1.8 ms | 3.6 ms | 33% |
| H200 | 6.0 ms | 2.9 ms | 3.1 ms | 49% |

Our serving energy-per-byte coefficient is pure memory-streaming energy. The proxy kernel costs 1.251e-10 J/B against 1.243e-10 from serving (ratio 1.006).

Our byte counts are right. Hardware counters read 1.004–1.008× the bytes our model assumes, in two separate captures. So the low bandwidth in serving is real underuse, not a counting error.

# 2. Is 200 ms sampling a problem? (square-wave power test)

What we ran. An on/off load at periods from 0.1 s to 2 s, logged with both NVML power readings, then fitted with a moving-average model.

Figure 2. (a) Under a 0.5 s on/off load, POWER_INSTANT tracks every step, while GetPowerUsage (what our serving logger used) is nearly flat. (b) GetPowerUsage sees only 30–50% of sub-second power swings.

What it shows. GetPowerUsage is a 0.85-second moving average on B200 (1.0 s on A5000), more than 4× our 200 ms bin. POWER_INSTANT is a 0.10 s average. Total energy is conserved, so J/token predictions are unaffected: within 2–5% over whole runs, with ≤ 1% bias. What breaks is assigning energy within a run to compute vs memory (section 3). From now on we log POWER_INSTANT and calibrate with long steady windows.

# 3. What the corrected coefficients are (GEMM loops + whole-run fits)

What we ran.

Steady loops. Qwen2-7B layer-shaped matrix multiplies (GEMMs) at 1–8192 tokens, as 10 s steady loops. Points at the power cap are excluded, because throttling distorts energy per FLOP.

Whole-run refit. The same energy model refit on whole-run totals (45 s windows), which averages out the smoothing.

Figure 3. Energy coefficients from the September per-bin fit (grey), the whole-run fit (blue) and the steady loops (green). On B200 the two smear-free methods agree.

What it shows. On B200:

Memory energy per byte is unchanged: 1.22–1.24e-10 J/B under every method.

Compute is 0.93–0.94 pJ/FLOP, not 0.64. Smoothing had pushed prefill energy into the wrong term.

A KV-cache byte costs about the same as a weight byte (1.03–1.09e-10), not 2.3×. That 2.3× was a smoothing artifact. Physically this makes sense: both are DRAM reads.

H200 can only be partly corrected: compute rises to 0.95–1.10 pJ/FLOP, but KV stays at 2.4× a weight byte. Settling it needs the steady test, and we no longer have H200 access.

# 4. Overprovisioned or balanced? (all 110 serving runs + steady data)

What we ran. Each run's energy split into idle, weight bytes, KV bytes and FLOPs; same-workload H200 vs B200 comparisons; and a power-limit "roofline" built from the steady data.

Figure 4. (a) Share of energy by component, 7B models. Idle power is 23–33% of energy on H200 and 36–45% on B200. (Panel (a) uses the September coefficients; with the corrected ones, GEMM's share grows.) (b) On the same workloads, B200 delivers 0.94–1.29× H200's throughput despite 1.67× the bandwidth, and costs 1.11–1.50× the energy per token.

What it shows.

At the loads we ran, B200 is overprovisioned against H200. It has 2× H200's idle power (238 W vs 116 W). Fixed overhead caps its throughput gain (section 1), so the extra idle power is never paid back.

Bandwidth is the capability worth paying for; compute is not, for decode. On H200, at ≤ 50 ms per token:

Doubling FLOPs saves under 1.5% of energy per token, which would justify only 4–8 W of extra idle power.

1.5× bandwidth saves 6–20%, which would justify 42–190 W.

Full B200 capabilities would justify 108 W (7B) to 240 W (32B) of extra idle; B200 actually carries 121 W. So B200 is marginally overprovisioned for 7B and justified from 14B up.

B200's compute is provisioned about 2.8× beyond its power budget. Peak FLOPs would need about 2.1 kW above idle against a 765 W budget. Every prefill-sized GEMM runs at the power cap, reaching only 50–58% of peak FLOPs.

The energy balance point is 133 FLOP/byte, against a performance balance point of 281. Decode runs at 1–90 FLOP/byte, so its energy is memory-dominated throughout.

Small models are the clearest overprovisioning. A 0.5B model on H200 uses 5% of bandwidth and spends 68–80% of its energy on idle power.

# Other updates

LIMINAL's "5×" is a latency gap, not a power gap. An H100 matrix-vector kernel (GEMV) was predicted at 146 µs and measured at 736 µs. The gap was cut from v2 and replaced by a fitted 114 µs/layer overhead, which matches our measured ~113 µs/layer. LIMINAL never measured power; it assumes TDP.

The 1/bandwidth energy law is now refuted for the DRAM channel too. We measured per-channel energies on B200: DRAM 8.4e-11, L2 2.3e-11 and L1 1.0e-11 J/B. These rebuild the serving coefficient to within 3%. For the law to hold, H200's DRAM alone would need 130% of H200's total per-byte energy.

Recommender: J/token error is unchanged (4.1% on H200, 3.7% on B200). The A100 memory prior was revised using EnergAIzer (ISPASS 2026).

# Next steps

Adopt the corrected B200 coefficients, and model the power-capped regime for prefill: at the cap the effective cost is about 0.6 pJ/FLOP.

Rerun B200 serving at concurrency 128–512. The model predicts that B200 overtakes H200 only at very large batches.

Log POWER_INSTANT in all future serving runs.